Assessment of the Availability

Of Natural Gas

In the

Gulf Coast Area

Edward H. Feinstein

- Introduction

Edward H. Feinstein has prepared this report on conventional natural gas supplies of the Gulf Coast Area.  In this report, specific reviews were made of the history, gas production, estimates of proven reserves and estimates of undiscovered resources.

The principal purpose of this report is to present estimates of the availability or productive capability of natural gas in certain regions of the Gulf Coast Area.  An assessment of the unconventional resource, gas shales of the Haynesville formation and coal-bed methane in the Gulf Coast Area is also included in this report.  Forecasts of the area-wide natural gas productive capability were based upon estimates of proven reserves, discovery process estimates of reserve additions, pipeline connection parameters and deliverability profiles.  Discovery process is the relationship between the efforts (drilling) and the potential for natural gas discoveries.

- SUMMARY AND CONCLUSIONS

The gas supply regions of the Gulf Coast Area are in both an intermediate and mature stage of development.  However, there are some promising new areas of exploitation.   The assessment of gas supply herein is based on three ingredients: remaining reserves, reserves appreciation and undiscovered resources.  Remaining reserves are the proved and economically producible gas discoveries.  Reserves appreciation is resources believed to exist that are directly related to reserves already discovered.  Undiscovered resources are estimated gas accumulations that are believed to exist, but have not yet been proven by drilling.

The productive capacities of proven gas reserves of each producing region of the Gulf Coast Basin vary considerably.    Reserves-to-production ratios in each area presently are at their lowest level, reflecting only modest surplus pipeline gas.

Estimates of future annual gas discoveries were made employing a discovery - process model as described below.  Productive capacity decline rates were applied to determine the availability of gas from new supply sources.

The availability of supplies from future sources was added to the availability of current proven sources to arrive at the overall productive capability of natural gas supplies from the various areas.

These supply areas are currently reliable, active and viable in providing adequate throughput for the network of pipelines connected to them.  In the long-term, however, the current grade of natural gas accumulations will be exhausted, giving way to the discovery of smaller deposits and the exploitation of unconventional, technologically sensitive hydrocarbon deposits.  The result will be a short-term increase in the production of gas throughout the basin then a gradual decline in the productive capability from existing and future connected supply sources.

III(A).  BACKGROUND – GULF COAST AREA

The Gulf Coast Area is made up of the states Texas, Louisiana and Mississippi, and the Gulf of Mexico.  The Gulf Coast Area remains the dominant producing area of natural gas in the United States.  Many of the reservoir rocks are highly porous and permeable, thus, capable of producing large volumes of gas.  Most of the area is, however, in a mature exploration stage.  Important new fields, nevertheless, continue to be discovered.  The Gulf of Mexico deepwater and the eastern Gulf shelf are still in relatively early stages of exploration.  Onshore areas are benefiting from improved drilling and producing technologies.  One example is the low permeability gas plays, including the Jurassic Haynesville shale gas in northeast Texas and northwest Louisiana.

A challenge for certain gas resources in the onshore regions is to exploit technically available gas in locations where reserves are characterized by  “tight” matrix porosity and permeability, naturally fractured reservoirs and coalbed methane and make them economically recoverable resources.

III(B).  BACKGROUND – GULF COAST AREA - UNCONVENTIONAL

Gas resources that now make up the category referred to as unconventional were always included in the analysis of ultimate resources in a particular area.  Only in “sweet spot” areas were these resources economically producible.  Most of these resources, however, were confined in unfractured shale matrix and tight sandstone formations, referred to as continuous-type deposits.  Sweet spots in gas shale formations are areas where the formation was naturally fractured.  Wells that penetrated these sweet spots along with induced hydraulic fracturing techniques resulted in very lucrative gas production.

Hydrocarbon accumulations in these deposits involve low exploratory risk because of the large known in place resource.  However, there remains a high technical risk.  The reservoir properties & productivity vary from play to play with low porosity, permeability & recovery factors.   Natural fractures are often critical for economic recoveries.  The formations may be over or under-pressured and these continuous-type deposits lack well-defined downdip water contacts.  After “flush production”, production levels off to much lower level , yet for a relatively long period.  The exploitation of such resources is conducive to assembly-line operations.

The Gulf Coast area contains one such promising shale gas resource, the Haynesville shale.  The Haynesville is an emerging play, that is, minimal production so far, but the initial productivities have been higher than observed in other shale plays.  Approximately 30-40 wells have been drilled so far (through 2008) with high initial production rates.  The wells are usually 10,000 to 13,000 feet to total depth, with shale thickness typically ranging from 100 to 300 feet.

IV.	METHODOLOGY

Proven Reserves

An analysis of the producibility of proven gas reserves was made using information obtained from the Energy Information Administration (EIA) and the Potential Gas Committee (PGC).   EIA’s proven reserves are as of the end of 2007.  The productive availability of those proven reserves was obtained from data assembled by the PGC and IHS and extrapolated employing a constant percentage decline until the reserves are exhausted.   The proven gas reserves were obtained from EIA, which in turn collected the data from producers.  The PGC and HIS provided the production rate of those reserves.

Future Reserve Additions

A characteristic observed in the petroleum producing areas of the North American gas supply areas is a rapid drop off in size from the largest known field to the smaller ones.  Hydrocarbon accumulations are the result of complex geological processes.  Furthermore, the actual quantities of producible reserves are further defined on the basis of technological and economic considerations.  As a consequence of all these independent influences and the multiplicative nature of the factors affecting the size of a gas accumulation, field sizes in producing basins are typically log normally distributed (Figure 1).

That is, a few very large fields contain the bulk of the reserves and many, many small fields contain, in aggregate, a smaller portion of the reserves.  Also, another characteristic of gas supply basins is that large fields are discovered early in the exploration process, and subsequent discoveries are smaller and the product of increasingly greater efforts.  This is demonstrated in illustrative form in Figure 2, below.

Since some of the regions in the Gulf Coast Basin, unlike other producing regions, contains both mature and intermediate supply regions, perhaps some large field discoveries remain undiscovered and will become available for exploitation and some portion of resource estimates may prove to have been too optimistic.

The Finding Rate Methodology

One measure of the discoverability of resources is the rate at which resources are found.  This method compares the drilling footage in a particular year with the related discoveries.  This method depicts the normal stage of events that take place when a gas-bearing province graduates past its initial discovery stage and enters its more or less mature stage.  The degree of maturity of the producing life of the supply areas can be determined by comparing the amount of gas resources already discovered with an estimate of the ultimate resources.

The nature of oil and gas accumulations creates a distribution of fields and reservoirs made up of a small number of large fields, a larger number of medium size fields and a seemingly unending amount of small fields.  The Gulf Coast Area is no exception.  An example of the distribution of gas reserves in the a portion of the Gulf Coast Area, the Gulf of Mexico, is shown on Figure 1.  This is typical of the distribution of gas reserves in an oil and gas province.

The basic concept of this Finding Rate Methodology is shown on Figure 2.  At times, the declining rate of effectiveness is mitigated by: better technologies for discovery and resource recovery, greater understanding of the geophysics, and reservoir performance of the field in the province.  This mitigation is also shown on Figure 2.

Advances in technology are, however, a double-edged sword with respect to extending the life of gas resources and ultimately the life of associated producing equipment and pipeline facilities.  Exploration and production (E&P) technology varies throughout the industry, from increasing the success ratio in exploration and, in the case of unconventional resources, exploitation to more efficient production techniques.  While some advances in technology may allow the commercialization of heretofore unproduceable hydrocarbon deposits, most others relate to the profitability of technically discoverable oil and gas resources.

An example of the effect of new geophysical technology (e.g. 3-D seismic) on E&P is basically an improvement in the exploration success ratio.  With advances in geophysical technology, producers are better able to locate oil and gas deposits and also to determine whether they should be explored or bypassed as a viable project.

Technology advances do not come cheap.  Its application must be in terms of the potential value of the resource.  This assessment takes into account technology, in that the forecasts were based upon the employment of various trends, which included advances in technology.

I first determined if the supply areas paralleled the premise of this model (that large initial field discoveries give way to smaller ones).  In addition to the field size facts cited earlier, further analysis confirmed that indeed most of the larger fields have been discovered as well as many of the medium size fields. This can be observed by inspecting the relationship between the new fields discovered over the years with heightened exploratory efforts in the Gulf Coast Area as shown on Figure 3, below.

This can also be seen by analysis of the finding rate methodology in terms of exploratory effort.  Most of the significant gas discoveries are actually associated with fields previously discovered.  See the historical data shown on Tables 1 and 2, and Figure 3.  The exploratory effect is the accumulation of wells drilled over time.  The above finding rate data is a 5-year snapshot of a long trend from higher levels of how effective exploration and development was in prior years.  I observed both exploratory wells and development wells.  Development wells do not reflect the effort to find new discoveries.  However, they contribute significantly to the reserve base.  “Results” (in terms of annual gas discoveries) of the drilling effort are also shown on Tables 1 and 2.

When these “results” or annual gas discoveries are divided by the annual exploratory wells drilled, a more focused relationship develops as to the size of the discovery for the effort expended.  This confirms that the large fields have already been discovered and that new discoveries are going to be generally confined to a considerably more moderate size.  This concept of discoveries per well drilled is referred to by the EIA as the Finding Rate Methodology.

The Finding Rate Methodology began in the late 1950s and early 1960s and continues to be used today.  The famous oil and gas forecaster, M. King Hubbert developed various aspects of it and used it in his presentations and forecasts.  The renown petroleum engineer and recipient of the C. C. Uren Award from the Society of Petroleum Engineers, J.J Arps also developed the Finding Rate Methodology in the early 1960s, referring to it as the Effectiveness of Exploration.  The methodology was and still is employed widely by those forecasting oil and gas resources.  I employed the methodology in 1973 and continue to do so.  The EIA exclusively uses the Finding Rate Methodology to forecast long range oil and gas discoveries in its state-of-the art Annual Energy Outlook publication.

The model used the relationship between annual reserve additions and both exploratory and development well drilling over time in years and cumulative feet drilled from a base of 1990.  For the most likely case, I extrapolated the exploratory finding rate at a constant level using the 2003 - 2007 (5 year) mean value developed in Tables 1 and 2 until a point is reached where 90 percent of the total endowment is reached.  The total endowment is defined as all the gas that will eventually be discovered (past discoveries plus the PGC’s estimates of potential resources).  PGC’s estimates of potential gas resources for the Gulf Coast Area are shown on Table 1.

Table 2 shows the total endowment as of 2006 for the Gulf Coast Basins of Texas, Louisiana and Mississippi.

I used the same procedure for the finding rate of development drilling.

The most likely level represents the mean value of the finding rate from 1990 through 2007.

I employed a constant level of effectiveness until 90 percent of the ultimate resources are discovered as I expect some occasional increases in the finding rate due to forces not directly indicated in the data.  As mentioned earlier, any decline in the finding rate curve will be mitigated by technological increases in the exploration and drilling techniques along with an increased awareness of the geophysics and reservoir mechanics.  Technological increases are included in the 1990-2007 data.  I am assuming that future technological increases will occur at the same rate as in the historical statistics.

I determined the future discoveries from exploratory drilling by applying a representative constant level of drilling activity to the corresponding finding rate.  For my determination of the discoveries from development drilling, I also applied a constant level of annual drilling activity, based upon the level of the most recent period, to reflect the development drilling activity response to increases in the wellhead price of gas.  This period included very significant increases in the price of gas at the wellhead.  I believe that in the future such similar increases and decreases will occur eventually leading to a further overall price increase.  My choice of exploratory and development drilling levels fully reflects an overall average price increase over the pertinent period, all the while daily, monthly and yearly prices will fluctuate both up and down.  Specifically, based on my experience and studies, I found a relationship to exist between the price of gas at the wellhead and development drilling effort.  No such clear relationship occurs for exploratory drilling as drilling prospects differ considerably in many respects as well as inherent risk factors.  As such, many factors come into play with respect to the exploratory drilling response.  While an increase in wellhead gas prices is an inducement to increase exploratory drilling efforts, the fact is that for the producing areas involved in this proceeding, there is no clear and concise relationship between wellhead price and the number of exploratory wells drilled.  The graphs shown on Figures 4 and 5, of wellhead gas price and drilling effort, illustrate this point.

Exploratory wells differ considerably from development wells in the Gulf Coast areas.  Exploratory wells are relatively high risk.  They are drilled relatively far from existing discoveries.  They are high cost.  Existing, in place, pipeline facilities may be lacking.  They must rely upon financing much different from development wells, e.g., the expenditure of money for geological and geophysical studies.  Many factors affect the decision to drill exploratory wells, including, but not exclusively, the prevailing wellhead price.

With respect to development wells and price, the annual relationship between them is not sufficient to forecast future drilling efforts.  Instead, I employed high values of such efforts in my calculations.  The Most Likely Case level of wells drilled and footage attained was based on the 2007 level.

The Future Discoveries resulting from the application of the drilling effort to the effectiveness of drilling in the Onshore Texas Gulf Coast region are shown on Table 3 for exploratory discoveries and Table 4 for development discoveries.

To determine the future gas availability, I applied to each determined annual future reserve addition, a production rate derived by the Potential Gas Committee from gas production data obtained from Petroleum Information/Dwights LLG (See Figure 6).

This results in the production capacity from new reserves beginning in 2008.  I applied the same production rate profile to each future amount of gas discoveries.  Actually, because of the progressively lower grade of gas deposits found in the future; and the new technology trending towards achieving faster revenue payouts, I expect the decline rate of the production rate profile to become steeper.  This would tend towards faster depletion of the future resources and eventually shortening the life of the endowment of gas in those areas.  By employing the current production profile decline rate to each increment of future discoveries, the results are somewhat conservative.

To the production profile of future reserves, I added the production profile for the beginning of year 2008 proven ( already discovered) gas reserves.  This production profile of the availability of natural gas in the Onshore Texas Gulf Coast is shown on Figure 7.

Similar determinations were made for the Onshore Louisiana region and the Gulf of Mexico.  Information on gas production, reserve additions and wells drilled in those areas was obtained from the same sources as the Onshore Texas Gulf Coast Area.  Availability profiles for the Onshore Louisiana Area and the Gulf of Mexico are shown below in Figures 8 and 9.

The serious nature of the decline in availability of gas in the Gulf of Mexico is supported by the current and recent historical decline in production.  This is shown in Figure 10.

An assessment was also made for the coalbed methane play in the Black Warrior Basin.  This is shown in Table 5, below.  No appreciable gas resources are indicated by the PGC beyond the 2007 assessment of proven reserves.

Table 5

Exhibit No. SNG-29

Year

Reserves

Production

Bcf

Bcf

1989

537

23

1990

1,224

36

1991

1,714

68

1992

1,968

89

1993

1,237

103

1994

976

108

1995

972

109

1996

823

98

1997

1,077

111

1998

1,029

123

1999

1,060

108

2000

1,241

109

2001

1,162

111

2002

1,283

117

2003

1,665

98

2004

1,900

121

2005

1,773

113

2006

2,068

114

2007

2,127

114

Coalbed Methane

Alabama

V.	DETERMINATION AND RESULTS  --  	GULF COAST AREA

The Gulf Coast Basin that I analyzed occupies the states of Texas, Louisiana and Mississippi.  This basin includes the Gulf of Mexico.  This is one of the major oil and gas producing regions of the United States.  Gas production will come from mostly non-associated gas reservoirs with about 10 percent from associated-dissolved gas sources (oil fields).  As these reservoirs become less prevalent, harder to find and uneconomical, new, more costly, yet potentially highly productive unconventional resources will be sought.  New field discoveries are expected to be found in deposits ranging from 1 to 200 Bcf, with most in the 2 to 20 Bcf range.  The profile of the future productive capacity from this area is shown in Table 6, shown below.

Table 6

Exhibit No. SNG-29

Profile of Availability of Natural Gas Reserves and Resources in the Gulf Coast Area

Onshore

Potential

Onshore

Gulf of Mexico

Black Warrior

Total

Year

Louisiana

Haynesville

Texas Gulf

Basin

5 Areas

Traditional

Shale Gas

Coast

Coalbed Methane

Gulf Coast Basin

2008

8.46

2.98

6.20

7.55

0.31

25.50

2009

8.67

3.60

6.97

5.68

0.31

25.23

2010

8.24

4.11

6.97

6.39

0.31

26.02

2011

7.65

4.57

7.07

6.14

0.31

25.74

2012

7.11

6.23

6.97

6.21

0.31

26.83

2013

6.49

5.87

6.57

6.17

0.31

25.42

2014

5.92

5.93

6.08

6.10

0.31

24.35

2015

5.35

6.07

5.55

6.21

0.31

23.50

2016

4.92

6.22

5.00

6.35

0.31

22.80

2017

4.48

6.24

4.57

6.23

0.31

21.83

2018

4.05

5.78

4.11

6.13

0.31

20.39

2019

3.69

4.92

3.70

5.88

0.31

18.50

2020

3.34

4.38

3.33

5.58

0.31

16.95

2021

3.02

3.95

3.00

5.22

0.31

15.50

2022

2.72

3.58

2.69

4.83

0.31

14.13

2023

2.45

3.24

2.41

4.43

0.31

12.84

2024

2.19

2.93

2.16

4.05

0.31

11.64

2025

1.95

2.65

1.93

3.68

0.31

10.52

2026

1.73

2.40

1.73

3.34

0.31

9.51

2027

1.56

2.17

1.55

3.02

0.31

8.61

2028

1.40

1.96

1.39

2.73

0.31

7.79

2029

1.25

1.78

1.24

2.47

0.31

7.05

2030

1.12

1.61

1.11

2.23

0.31

6.38

2031

1.00

1.46

1.00

2.02

0.31

5.78

2032

0.90

1.32

0.89

1.82

0.31

5.25

2033

0.81

1.19

0.80

1.65

0.31

4.76

2034

0.72

1.08

0.72

1.49

0.31

4.32

2035

0.65

0.98

0.64

1.35

0.31

3.93

2036

0.58

0.88

0.58

1.22

0.31

3.58

2037

0.52

0.80

0.52

1.18

0.31

3.33